Inductance Exam

Answer Key

Match the terms with their correct definitions:

| 1. | Inductor | C | A The property of an electric circuit when an EMF is induced in that |
| --- | --- | --- | --- |
|  |  |  | circuit by a change of current |
| 2. | Inductance | D | B The unit of inductance |
| 3. | Self-inductance | A | C A device that transfers changing current and voltage from one circuit |
|  |  |  | to another by inductive coupling |
| 4. | Henry | B | D The property of an electric circuit when a varying current induces an |
|  |  |  | EMF in that circuit or another circuit |

Match the terms with their correct definitions:

| 5. | Permeability | D | A A number indicating the fraction of flux lines of one circuit cutting |
| --- | --- | --- | --- |
|  |  |  | another circuit |
| 6. | Mutual inductance | C | B A device that transfers changing current and voltage from one circuit to another by inductive coupling |
| 7. | Coupling coefficient | A | C The property of two circuits whereby an EMF is induced in one circuit by a change of current in the other |
| 8. | Transformer | B | D The measure of ease with which material will pass lines of flux |

Match the inductance unit abbreviations with their correct definitions:

| 9. | L | A | A Inductance |
| --- | --- | --- | --- |
| 10. | k | D | B Henry |
| 11. | di/dt | C | C Rate of current change |
| 12. h |  | B | D Coefficient of coupling |
| 13. µ |  | E | E Permeability |

- Which of the following statements is true concerning Lenz’s law?

A  Induced voltage is called electromotive force (EMF)

B  Induced voltage is so important that it is the basis of Ohm’s law

C  Induced voltage is called counter electromagnetic input (CEMI)

D  Induced voltage at every instant opposes any change in circuit current

- Which of the following statements is true concerning factors affecting inductance of coils?

A Number of turns- Inductance varies directly with the square of the number of turns

B Permeability of core- Inductance varies inversely with the permeability of the core

C Cross-sectional area of core- Inductance varies inversely with the cross-sectional area of the core

D Length of core- Inductance varies directly with the length of the core

- Which of the following does not identify this kind of inductor?

A powdered iron

B air

C ferrite

D iron

Match the formulas for total inductance of inductors connected in series or in parallel to their correct definition:

| 17. | Two inductors in parallel (different values) | B | A | LT = L |  |  |  | + L + L3 + … |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. | Three different value inductors in parallel | C | B | LT = |  |  | L1 x L2 |  |  |  | = |  |  |  |
|  |  |  |  |  |  | L1 + L2 |  |  |  |  |  |  |  |  |
| 19. | Three inductors of same value in series | A | C |  | 1 = 1 + 1 + 1 = |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  | L1 L2 L3 |  |  |  |  |  |  |
| 20. | Three inductors of same value in parallel | D | D LT = L / N = |  |  |  |  |  |  |  |  |  |  |  |

Match the formula to what it is used for:

| 21. | = CEMF |  |  |  |  |  | C | A solves for mutual inductance opposing |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | di/dt |  |  |  |  |  |  |  |
| 22. | = k √ |  |  |  |  |  | E | B solves for mutual inductance aiding |  |
| 23. | = L1 |  | + L2 + 2LM |  |  |  | B | C solves for Henry |  |
| 24. | = L1 |  | + L2 – 2LM |  |  |  | A | D solves for turns ratios |  |
| 25. | = NS / NP |  |  |  |  |  | D | E solves for mutual inductance |  |

- Which of the following statements is true concerning transformer ratios?

A In the step down transformer, NP is smaller than NS.

B Turns ratio is the ratio of number of turns in secondary winding to number of turns in the primary winding (NS/NP).

C Current ratio, IS/IP, equals voltage or turns ratio.

D Power ratio, PO / PI, power in is less than power out.